Subject: Micro/WIN 4.0 SP9 installation problem with a newer version of S7DOS already installed on Win7 x64

FYI. We have been receiving reports of install problems for Micro/WIN V4.0 SP-9. Customers report no PC/PPI cable driver appears in the list of communication drivers after the install completes. Here is a work around to address this problem.

Mack

This problem only occurs on a x64 machines and only when a newer version of S7DOS is installed (newer than what MicroWIN 4.0.9.25 currently has).

The attached bat file will fix this issue.

It runs the cmd to update the XML for Simatic OAM.

Unzip it into any folder.

Run it.

If UAC is on, it will prompt to elevate, do so.

A cmd window will open for as long as it is running.

When complete, the cmd window will go away.

Verify by running Set PG/PC Interface.

