Time, Date and Timer in STEP 7

TIME, DATE and TIMER in STEP 7
SLC A&D CS
June 2004
31.
西门子自动化与驱动产品的在线技术支持


42.
如何获得西门子自动化与驱动产品的资料


43.
需设备选型及订货


44.
西门子技术支持热线


55.
西门子自动化产品的其它网站


56.
缩写词含义


57.
如何使用STEP 7软件的在线帮助


57.1.
查找某个关键字或功能


67.2.
想了解某个FB/FC的功能及管脚的定义


88.
STEP 7中有关时间、日期的数据类型


89.
需要相关数据类型的详细描述


910.
S5TIME (Time Duration) 的格式及访问


910.1.
S5TIME的格式


1010.2.
通过上位机访问PLC中S5TIME类型的数据


1110.3.
S5TIME和TIME的转换


1111.
DATE_AND_TIME的格式


1312.
STEP 7相关功能块调用例程


1413.
STEP 7中定时器的使用


1413.1.
S7 TIMER的使用


1413.2.
IEC TIMER的使用


1414.
如何访问CPU的Clock Memory




1. 西门子自动化与驱动产品的在线技术支持
首先,建议您访问Siemens A&D的产品与技术支持网站 http://www4.ad.siemens.de/ 

[image: image1.jpg]icrosoft Internet Explorer

Fle Edt View Favortes Took Hel

ok - 5 - D @) Q| Qewch (iravortss Gveds 3| B b

sz | g jwnnt b e el

SIEMENS mens.com Intranet
International Automation and Drives il (EEon e SRl Cortect | Stemso

Automation Home | Product Support | Services | Information | Support Shop BSTIME
and Drives
Service & Support

(T LinkBox
B i one mouse ek to
further iformaton

arounthe prodct

> Abautus

mySupport > What is 17

Inthe area Praduct Support, you will ind all documents regarding our products and
systems, like tips and ticks (FAG), downloads, manuals and certicates. With the
help ot these documents we assistyou in inding a solution to your technical

o DConmitot qUESHIONS. 210 prsonelzston
Productrange Docurenttype
> Dive Technolg > Sensor o > Menuls 7] > Support Reatest
5 Automation systems > Power suppies > Downloads [ | | coriarts wordwide
5 aion Networ > Satet inecrated > Ceticate

<= Techrical Forum
peh o for branches

> Brocess automation > Fisd Service

Whetheryou need a service
specialist, an expert for
consulation or a spare part We
offeryou the right support in every

Hers you willfind a
comprehensive offer for realizing | (e Newsleter
your automation project faster and

more effciently in all phases. =

> Support Shop - Survey

> System lities > Software rchiv
> Technicel Support > Appications > Automation Value Card

[ [ [BE Localintranet

]
Hstart[|| @ B) A © || Bl | By | o] Sa.| £36.[[E)s As.. | B | B | Bs. | &L L4 2R =r>QS@E e





在主页的Product Support中选择您关心的产品或信息，或利用Search引擎直接输入关键字搜索。
您还可以访问西门子（中国）自动化与驱动集团的主页www.ad.siemens.com.cn ，如下图所示。该网站同样可以提供大量的产品和系统的信息。

[image: image2.jpg]rosoft Internet Explorer

Fle Edt View Favortes Took Help

o - DB Q] Qewch (revoss Gveds 3| B b
@

siemens.comcn/ervice]

B SR KE
BEF (PE)

EREHERS

LBRRFETE
> BREFAE - RNERRSSRE - WESR
BARREE

FERERNER

RS

> ERERESEE - SARSRETE

- RAKR - SRER
-axAE WEF -paRe

o
WinCC
S » BIVF B SRRSO SR N FITL PR » Tl #

FRIVEHNIR. HEAFS, BRRT TSR ER SRS

EPE, E L oAt —

bR S ]
& [ [ [ et
Hstart[|| @ B) A © || Bl | Byd...| ko] L. | Ban | 5L | s | £0a.| PBa. | E0E.[[ETH. 2 R = » @ SR sszen





2. 如何获得西门子自动化与驱动产品的资料

首先，建议您通过Siemens A&D的网站搜索并下载。

您还可以致电010-64721888转3785 /3726索取资料。

另外，还有大量的手册可以通过分销商订购，和其它产品的订货方式一样。

3. 需设备选型及订货

如需设备选型及订货，请联系西门子自动化与驱动的销售工程师或当地西门子分销商。分销商联系方式可致电010-64719990获得。

4. 西门子技术支持热线

如有无法自行解决的技术问题，请拨打西门子技术支持热线010-64719990登记，等待西门子技术支持工程师回复。我们会在8小时内予以响应。

请注意在登记问题时尽量准确地描述所使用产品的类型，以便尽快得到负责该产品的工程师的帮助。

技术支持传真：010-64719783。

技术支持邮箱：adscs.china@siemens.com。

5. 西门子自动化产品的其它网站

www.s7-200.com 提供S7-200 PLC相关知识及软件下载。

www.mall.ad.siemens.com 查找西门子自动化与驱动的所有产品订货号、图片、及技术参数。
6. 缩写词含义

IEC：International Electrotechnical Commission，国际电工技术委员会

FAQ：Frequently Asked Questions，常见问题解答

7. 如何使用STEP 7软件的在线帮助

7.1. 查找某个关键字或功能

1、在STEP 7的主界面SIMATIC Manager中，点击下拉菜单Help下的Contents，打开STEP 7的在线帮助

[image: image3.jpg]K SIMATIC Manager

SIMATIC 300(1

Introduction
Getting started

Using Help
about.





2、利用Index进行关键字的查找

[image: image4.jpg]E? HTML Help
= 2

o @
Hide Back Foward Home Pt Glassay Objects

e

Objects and Object H
Type inthe kepord o find

In the same way that the Windows
aata e

projects and libraries in STEP 7 is

The following figure shows an exan

B paes

Station

Progr. Mool





3、或者，利用Search进行相关搜索

[image: image5.jpg]E? HTML Help

3 e = o &

Hide Back Foward Home Pt

Glossany_Objects

Cononc | e S0 |

Type inthe keyord to find:
fward

T TiTopes

Select Topic to display:

‘Accessing the Peripheral Data Aiea
Accessing the Process Data Area
‘Adding Associated Values to Messages
Addresses and Data Types Permited in
Assignment List

Objects and Object Hierarchy

In the same way that the Windows Explorer shows
projects and libraries in STEP 7 is shown in the S|

The following figure shows an example of an objec

B paes

Station

Progr. Mool





7.2. 想了解某个FB/FC的功能及管脚的定义

将您想要调用的FB/FC调入到一个Network中，选中该FB/FC（用鼠标点击该FB/FC，外框变为绿色），如下图所示：

[image: image6.jpg]HE{LAD/STL/FBD - [0B1 - S7_Pro1\SIMATIC 314C\CPU 314C-2 DP]
O Fle Edt Inseit PLC Debug View Optons Window Help

Dl(e-8] & slele] of - plalfs =] YOS & ol ) el
2] x|

i Contents Of:

=5 Inerface Nane
T Fea0 TONR TINERS S| & e

AFC2 SHRB. SHIFT fretork 7fESISTY
A FC93 SEG COMVERT

AT FC94 ATH CONVERT [Fomment:
AT FC5 HTA CONVERT

0 CONVERT
AT FC98 BCDCPL CONVERT TSCALE"
AT FC99 BITSUM COMVERT o

T FCI0L SETL BT {06 e il BELVALS 220
T FCI02 DBV MATH_Fs ks 1on ki
THFC103 COT TaBLE =
T FC104 TBLTBL TABLE
T FC105. SCALE. CONVERT
0 FC106. UNSCALE. COMVERT
@ Micelanous Blocks

HET CP

222L0_LTH

22, 2{BIROLAR

SIMATIC,





按您计算机键盘上的F1功能键，就会跳出关于该FB/FC的功能及管脚定义的描述。如下图所示：

[image: image7.jpg]‘Standard Function Help =100 x|
Fie Edt_pooknark_Options_fish
T e e I |

: FC105

The SCALE function takes an integer value (N) and coverts it to a real value in engineering units scaled between a low and a high limit (LO_LIM and
HI_LIM). The result is written in OUT. The SCALE function uses the equation

OUT = [ ((FLOAT (IN) O K1)(k2OIK1) * (HI_LMOILO_LIM)] +LO_LIM
| The canstants K1 and K2 are set based upon whether the input value is BIPOLAR or UNIPOLAR,
BIPOLAR: The inut integer value is assumed to be between C127648 and 27648, therefore, K1 = C127648.0 and K2 = 427648.0

 UNIPOLAR: The input integer value is assumed to be between 0 and 27648, therefore, K1

0and K2 = +27648.0

Ifthe input integer value is greater than K2, the output (OUT) is clamped to HI_LIM, and an error is retumed. If the input integer value is less than K1, the
output is clamped to LO_LIM, and an error is returned.

Reverse scaling can be abtained by programming LO_LIM > HI_LIM. With reverse scaling, the value of the autput decreases as the value of the input
increases.

| FC105 Parameters

Parameter Declaration Datatype Memory area  Description

EN Input BOOL 1GMDL Enable input with signal state
of 1 activates the
ENO Output BOOL 1GMDL Enable output has 2 signal

state of 1 ifthe function is
executed without error.

IN Input INT 1QMDLP,  The input value to be scaled to
! Constart a real value in engineering

' units
H_LIM Input REAL 1GMDLP,  Upper limit in engineering units.





您可以在该帮助信息中了解到该FB/FC的功能、参数的描述及所要求的数据类型、可能的错误信息等，有些还有例子程序。

8. STEP 7中有关时间、日期的数据类型

TIME：IEC time

S5TIME：SIMATIC time

DATE：IEC date

TIME_OF_DAY：time

DATE_AND_TIME：DT

各数据类型占用字长、格式及范围如下：

[image: image8.jpg]Type and
Description
M

(SIMATIC time)

(IEC time)

Data Type

in Format Options Range and Number Notation (lowest to  Example
ighest value)_

1B S7timein S5TAOH_DM_0S_10MS to L S5T#OH_1M_0S_OMS
eps o S5T#2H_4BM_305_OMS and

10 ms (default) SETHOH_OM_05_OMS SETIME#DH_1H_1M_0S_OMS
S P e L T#OD_1H_1M_0S_OMS

integer sitned T#24D 20H 31M 235 BATMS LTIME#ID 1H 1M 05 OMS
16 IEC date in steps of 1fBay D#1990-1-1 to L D#1996-3-15

168-12:31 L DATE#1996-3-15
%2 Timeinstepsof iffls  TODAIO0Oto LTOD#:1033
TOD#23:59:59.999 L TIME_OF_DAY#1:10:3.3

Description

Defines an area with B4 bits (8 bytes). This data type saves in binary coded decimal format




9. 需要相关数据类型的详细描述
请在STEP 7的主界面SIMATIC manager中的Help下拉菜单中点击Contents，在下图所示的帮助界面中利用索引Index搜索关于相应的数据类型的描述。

[image: image9.png]=loix|
I /A ©

Hide Back Foward Home Pt Glassay Objects

ontents | Iodex | Searc =l
Cortent = )
T e ot g Elementary Data Types
ype i the keyord o i
. Each elementary data type has a defined length. The following table lsts the elementary data types.
e
Type and in Format Options Range and Number Notation (lowestto  Example
ul [ Description highest value)_
Tine 3nd Date BOOL(EH) § Boolean text TRUE/FALSE TRUE
Seltng
lexadecimal number to
e e o Bl BYTE 8 Hexadecimal numb B0 to BHIGHEF L B#EHD
Tine Fomat (Byte) ? L byte#E#10
”‘"7 morioing during updat WORD 16 Biinary number L 2#0001_0000_0000_0000
- Word) ? > o ?
sclcive delay of pcte Hexadecimal number L W#1B#1000
TIME OF DAY 2 #1111 111111 L word#16#1000
Tine starp BCD V1540 to WHIBAFFFF L cioa
Tine g Corlits Decimal number unsigned ? L B#10.20)
Time stamps in UDTs and De (D EZED L byte#(10,20)
Time Stamps:as a Block Prop B#0.0) to B#255.255)
in ‘L"S‘a"?; Df‘aﬁ‘ﬂcks DWORD 2 Binary number 24010 2#1000_0001_0001_1000_
P & Bata Blocks (Double ward) ? 28111 111_1111_1111 1011_1011_0111_1111
Tine Zones ? AT 171 171 ?
Tine Reading Hexadecimal number  DW#E#00D0_00D0 to DWHIBAFFFF_FFFF L DWHHIBAI0A2 1234
Selting Decimal number unsigned B40,0,00) to L dword#IBA00A2_1234
ot semoutyitricmtss B#255 255 255,255) LB#(1, 14,100, 120)
ik L byte(1,14,100,120)
Fiules INT 16 Decimal number signed 32768 to 32767 L1
Dby nens (riege)
Tine of Day Irtemipt Ogariz DINT 32 Decimal number signed  L#2147483548 to 42147483647 Li#
el Dy ot i (Integer, 324its)
Py B 2 EEE L 1.234567e 13
Quenms (Floating- Floating-point number — Upper imit: *3.402823e+38
s point
Stating L |
TineofDaylrtempts ! || Mumbe) Lower limit: £1.175 4952-38
Handing SETIME B S7timein SETHIH_OM_0S_10MS to L S5T40H_1M_0S_OMS
TIMER (SIMATIC time) steps of ST#2H_4BM_305_OMS and L
Tiers 1) = 10 ms (defaul) SETHIH_OM_0S_0MS SSTIMEAOH_1H_1M_0S_OMS
TIME 32 IECtime in steps of 1 ms, “T424D_20H_31M_235_BABMS to L THID_1H_1M_0S_OMS
Disply (EC time) integer signed TH24D_20H_31M_235_BATMS L TIMEROD_1H_1M_0S_OMS

Rstort]| & 3 1 © || B Gy o] @] B0 8. ] B 1| B B [T ©w [BG 2EE=»BI@E s





10. S5TIME (Time Duration) 的格式及访问

10.1. S5TIME的格式

S5TIME是用BCD码保存的，在数据存储区占用两个连续的字节。下图给出的例子中时间值为127，时基为1秒。 
[image: image10.jpg]15 8 7 0
x 1x 1 to oo totsfoto fitofats 11ty

l 1 2 7
-
Time base Tirme value in BCD (010 999)

Inrelevant: These bits are ignored when the timer is started





当使用S5TIME时，定义数值的范围为0~999，而且要指明使用的时基。时基指定了时间单位。 
S5TIME时基及相应的时间范围如下：

	时基
	BCD码
	时间范围

	10 ms
	00
	10 ms - 9 s 990 ms

	100 ms 
	01
	100 ms - 1 min 39 s 990 ms

	1 s
	10
	1 s - 16 min 39 s

	10 s
	11
	10 s - 2 hr 46 min 30 s


您可以采用如下两种格式定义时间值：

·  1) W#16#wxyz 

· w = 时基 (即时间间隔或精度) 

· xyz = BCD格式的时间值 
这种方式只能用于STL编程。
· 2) S5T#aH_bbM_ccS_dddMS 

· a = 小时, bb = 分钟, cc = 秒, dd = 毫秒
· 这种情况下时基自动选择， 数值为该时基下取整去尾到下一个较低值。 

您可以输入的最大值为9,990 秒，或2H_46M_30S。

10.2. 通过上位机访问PLC中S5TIME类型的数据

如果想通过上位或触摸屏对PLC中S5TIME类型的参数进行设定，有如下方法：

1、从上位机写整型数INT或实数REAL到PLC，首先该数值需包含以毫秒为单位的时间值，在写入PLC的数据存储区后，利用ITD（Integer to Double Integer）或RND（Real to Double Integer with Rounding Off）将该值转换为双整形，然后将该值写到类型为TIME的变量里，在程序中调用FC40，将TIME转换成S5TIME即可。

请点击http://www4.ad.siemens.de/WW/view/en/11932644 参见在线FAQ。

2、从上位机写WORD到PLC，首先该数值需包含以某时基为单位的时间值，在写入PLC的数据存储区后，用Word Logic下的WOR_W指令将该值与其时基相或，再利用MOVE指令将得到的数值写入S5TIME类型的变量中。

3、如果使用WinCC作为上位软件，或上位软件支持32位带符号浮点数，可以从上位写32位带符号浮点数到PLC中定义为TIME的变量，然后在程序中调用FC40，将TIME转换成S5TIME即可。
例程参见
[image: image11.wmf]OB1.pdf

。

10.3. S5TIME和TIME的转换

FC33用于S5TIME到TIME的转换。FC40用于TIME到S5TIME的转换。这两个块位于Libraries\Standard Library\IEC Function Blocks路径下，关于功能块的描述详见在线帮助。
11. DATE_AND_TIME的格式
当使用数据类型DATE_AND_TIME (DT) 时，将占用数据存储区的8个连续字节，以BCD码格式保存。该数据类型范围如下： 
DT#1990-1-1-0:0:0.0 to DT#2089-12-31-23:59:59.999 
下面举例说明1993年12月25日上午8时1分1秒23毫秒，星期四的语法结构，有两种格式：

· DATE_AND_TIME#1993-12-25-8:01:1.23 

· DT#1993-12-25-8:01:1.23 

下列IEC标准功能块进行与DATE_AND_TIME相关的操作： 

· 将DATE和TIME_OF_DAY (TOD) 结合起来，转换成DATE_AND_TIME (DT)。该功能块的管脚IN1必须在DATE#1990-01-01 到DATE#2089-12-31范围内。
FC3: D_TOD_DT 

· 从DATE_AND_TIME 中取出DATE。
FC6: DT_DATE 

· 从DATE_AND_TIME 中取出the day of the week，即星期几。
FC7: DT_DAY 

· 从DATE_AND_TIME 中取出时间。 

FC8: DT_TOD 

下表给出了以1993年12月25日上午8点1分1秒23毫秒，星期四为例，在date and time 的八个字节中分别存储的内容： 
	字节
	内容
	例子

	0
	Year
	B#16#93

	1. 
	Month
	B#16#12

	2. 
	Day
	B#16#25

	3. 
	Hour
	B#16#08

	4. 
	Minute
	B#16#01

	5. 
	Second
	B#16#01

	6. 
	MSEC 的后二位
	B#16#23

	(高4位)
	MSEC 的高位
	B#16#0

	7 
(低4位)
	Day of week 
1 = Sunday 
2 = Monday 
... 
7 = Saturday
	B#16#5


DATE_AND_TIME 允许的数值范围如下： 

· 最小： DT#1990-1-1-0:0:0.0 

· 最大： DT#2089-12-31-23:59:59.999 

	
	允许的范围
	BCD 码

	Year 
	1990 - 1999 
2000 - 2089 
	90h - 99h 
00h - 89h 

	Month 
	1 - 12 
	01h - 12h 

	Day 
	1 - 31 
	01h - 31h 

	Hour 
	00 - 23 
	00h - 23h 

	Minute 
	00 - 59 
	00h - 59h 

	Second 
	00 - 59 
	00h - 59h 

	Millisecond 
	0 - 999 
	000h - 999h 

	Day of week 
	Sunday - Saturday 
	1h - 7h 


12. STEP 7相关功能块调用例程

在STEP 7在线帮助中提供了下述SFC/FC的调用例程。查找该例程的方法如下：将SFC0/SFC1添加到某一个Network中，按您计算机键盘上的F1功能键，在See also中，可以看到该例子程序，以及该程序的描述。具体如下：

在组织块OB1中调用FC10，在FC10 中设置时间并读出。被调用的SFC/FC如下：
·
SFC 0  "SET_CLK" 设置CPU时钟

·
SFC 1  "READ_CLK" 读出CPU时钟

·
FC 3 "D_TOD_DT" 

·
FC 6 "DT_DATE"

·
FC 7 "DT_DAY"

·
FC 8 "DT_TOD"
例程参见：
[image: image12.wmf]"sample program"

。

13. STEP 7中定时器的使用

13.1. S7 TIMER的使用

在STEP 7的Program elements中的Timers菜单下，提供了5种S7类型的Timer，调用方法同其它功能块。定时时间类型为S5TIME，访问方式见第10.章节的描述。每一款CPU所支持的S7类型的Timer的数量是一定的，具体数量参见产品手册。

13.2. IEC TIMER的使用

如果您在程序中药使用更多的定时器，建议您使用Libraries\Standard Library\System Function Blocks下的SFB3/SFB4/SFB5。SFB3~5使用的是IEC TIME，调用的次数计入该CPU所支持的Blocks的数量中。
14. 如何访问CPU的Clock Memory
在硬件配置时，需要设置CPU的属性。其中有一个选项为Clock Memory，选中选择框就可激活该功能，如下图所示：

[image: image13.jpg]— | Cyoki nteupt | Disgnostes/Ciock | Prtecton | Commuricaton
Gereral | Statup Cycle/Clack Memoy | Retentive Memayy | Interupts

B

¥ Updatz 081 process mage cycicaly

Sean Cyele Monitaing Tie s}

|| i

Sean Cycle i (s

Sean Cycle Load fom Commurication [}

Al

Size o he Process Imege

0885 - CallUp at 1/0 Access Eror: No 085 call up

Clack
| ¥ Clock memay
Mermory Byte:

:





然后在Memory Byte中输入您想为该项功能设置的MB的地址即可，如MB10，则输入10即可。

Clock Memory的功能是在所定义的MB的各个位周期性地改变其二进制的值（脉宽—间歇比例为1:1）。

Clock Memory的各个位的周期及频率如下：

	Bit
	7
	6
	5
	4
	3
	2
	1
	0

	Period duration (s):
	2
	1.6
	1
	0.8
	0.5
	0.4
	0.2
	0.1

	Frequency(Hz):
	0.5
	0.625
	1
	1.25
	2
	2.5
	5
	10


如果您在硬件配置里选择了该项功能，就可以在程序里调用，例如不同频率的方波。







































































































































SLC A&D CS                                                                    Page 2 of 15                                      


_1148719041.pdf


SIMATIC        timer_darst\S7-Programm(1)\...\OB1 - <offline>        09.05.2000 9:08:47


                                                                                Seite 1


OB1 - <offline>
"MAIN_SCAN"     
Name: Familie:
Autor: Version: 00.01


Bausteinversion: 2
Zeitstempel Code:


Interface:
09.05.00 09:08:37
08.05.00 13:00:42


Längen (Baustein / Code / Daten): 00164  00048  00022


Adresse Deklaration Name Typ Anfangswert Kommentar
0.0 temp OB1_EV_CLASS BYTE Bits 0-3 = 1 (Coming event), Bits 4-7


 = 1 (Event class 1)
1.0 temp OB1_SCAN_1 BYTE 1 (Cold restart scan 1 of OB 1), 3 (S


can 2-n of OB 1)
2.0 temp OB1_PRIORITY BYTE 1 (Priority of 1 is lowest)
3.0 temp OB1_OB_NUMBR BYTE 1 (Organization block 1, OB1)
4.0 temp OB1_RESERVED_1 BYTE Reserved for system
5.0 temp OB1_RESERVED_2 BYTE Reserved for system
6.0 temp OB1_PREV_CYCLE INT Cycle time of previous OB1 scan (mill


iseconds)
8.0 temp OB1_MIN_CYCLE INT Minimum cycle time of OB1 (millisecon


ds)
10.0 temp OB1_MAX_CYCLE INT Maximum cycle time of OB1 (millisecon


ds)
12.0 temp OB1_DATE_TIME DATE_AND_TIME Date and time OB1 started


Baustein: OB1  main scan cycle


Netzwerk: 1      on-delay timer T1


      U     "T1_Set" M0.0               -- 
      L     "T1_S5_time" MW2                -- 


      SV    "On-delay" T1                 -- 


      U     "T1_Reset" M0.1               -- 
      R     "On-delay" T1                 -- 
      L     "On-delay" T1                 -- 
      T     "T1_Integer" MW4                -- 
      LC    "On-delay" T1                 -- 
      T     "T1_BCD" MW6                -- 


Netzwerk: 2      data type conversion: WORD (T1_BCD) to S5TIME (BCD_S5TIME)


      L     "T1_BCD" MW6                -- 
      T     "BCD_S5TIME" MW12               -- 


Netzwerk: 3      convert S5TIME to (IEC) TIME


      CALL  "S5TI_TIME" FC33               -- 
       IN     :="BCD_S5TIME" MW12               -- 
       RET_VAL:="T1_IEC_time" MD8                -- 
      BE    






_1148800747.doc
//Addresses Used


//Address 
Meaning


//I      0.0
Input to read a variable time into MD 0


//I      0.1
Input to set the time


//MW 0
Date, can be changed via STATUS VAR


//MD  2
Time, can be changed via STATUS VAR


//MW 6
Current date


//MW 8
Current day


//MD  10    
Current time of day


//***********************************************************************************


FUNCTION FC 10 : VOID


TITLE = CPU- Set and read CPU time 


VERSION : 1.0


VAR_TEMP


IN_TIME
: TIME_OF_DAY ;
//TOD actual parameter


OUT_TIME
: TIME_OF_DAY ;
//TOD actual parameter


IN_DATE
: DATE ;
//DATE actual parameter


OUT_DATE
: DATE ;
//DATE actual parameter


IN_TIME_DATE

: DATE_AND_TIME ; 
//DT actual parameter


OUT_TIME_DATE

: DATE_AND_TIME ;
//DT actual parameter


CUR_RETVAL
: INT ;
//Current return value


END_VAR


BEGIN


NETWORK


TITLE = Save the required time and date


// in the temporary variables



L
D#1997-1-20 ;
//Specify date as constant 



AN
I 0.0 ;

//if I 0.0 = 0; variable value in MW 0 if I 0.0 = 1



JC
nko1 ;



L
MW 0 ;

//Specify value in MW 0 as date






//(change via STATUS VAR)


nko1:
T
#IN_DATE ;




L
TOD#13:30:0.0 ;
// Specify time as constant



AN
I 0.0 ;

// if I 0.0=0; variable value in MD 2 if I 0.0 = 1



JC
nko2 ;



L
MD 2; 

// Specify value in MD 2 as time


//(change via STATUS VAR)


nko2:
T
#IN_TIME ; 



NETWORK


TITLE = Combine date and time


// to the output variable (format DATE_AND_TIME)



CALL
FC 3

// Standard FC3 "D_TOD_DT" (conversion D and TOD






// to DT) 



  (IN1

:= #IN_DATE,
//Combines date from input variable (format DATE)



   IN2

:= #IN_TIME,
//and time from input variable 



   RET_VAL
:= #IN_TIME_DATE);
//(Format TIME_OF_DAY) 






//to form output variable






//(format DATE_AND_TIME)


NETWORK


TITLE = Set time at positive edge


A
I 0.1 ;

// Set time if positive edge at I 0.1



FP
M 255.1 ; 



=
M 255.0 ;

// Edge memory bit


NETWORK


TITLE = Set time at positive edge



A     
I  0.1 ; 
//Set time if positive edge at I 0.1



FP   
M 255.1 ; 





=
M 255.0 ;
//Edge memory bit


NETWORK


TITLE = Set the time 



A
M 255.0 ;

// Set time if M 255.0=1 (positive edge)



JNB
_005 ;  



CALL
SFC 0

// SFC 0 "SET_CLK" (set time-of-day interrupt)



  (PDT

:= #IN_TIME_DATE,
// Current date and time



   RET_VAL
:= MW 200);
// Contains error code (if SFC processing faulty)


_005:
A
BR; 



=
Q 4.0 ;

// Q 4.0=1 if setting time OK


NETWORK


TITLE = Read time



CALL
SFC 1

// SFC 1 "READ_CLK" (read time)



  (RET_VAL
:= #CUR_RETVAL ,
// Contains error code 






//(if SFC processing faulty)



   CDT

:= #OUT_TIME_DATE); 
// Contains read time






// and date (format DT)


NETWORK


TITLE = Extract date from format DATE_AND_TIME



CALL
FC 6

//FC6 "DT_DATE" (conversion DT to DATE)



  (IN

:= #OUT_TIME_DATE,
// Read time and date






//(format DT) 



   RET_VAL
:= #OUT_DATE); // Output variable in format DATE






//(current date)


NETWORK


TITLE = Display current date



L
#OUT_DATE ;



T
MW 6 ;

// Display current date in MW 6


NETWORK


TITLE = Extract day from format DATE_AND_TIME



CALL
FC 7

// FC7 "DT_DAY" (conversion DT to DAY)



  (IN

:= #OUT_TIME_DATE,
// Read time and date 






//(format DT)



   RET_VAL
:= MW 8)
// Display current day in MW 8


NETWORK


TITLE = Extract time from format DATE_AND_TIME



CALL
FC 8

// FC8 "DT_TOD" (conversion DT to TOD)



  (IN

:= #OUT_TIME_DATE,  //Read time and date 






//(format DT)



   RET_VAL
:= #OUT_TIME);
//Output variable in format TOD






//(current time of day)


NETWORK


TITLE = Display current time



L
#OUT_TIME ;



T
MD 10 ;

//Display current time of day in MD 10


END_FUNCTION


//***********************************************************************************


ORGANIZATION_BLOCK OB 1


TITLE = Call FC10


VERSION : 1.0


VAR_TEMP



SYSTEM :
ARRAY [1..20] OF BYTE ;


END_VAR


BEGIN



CALL
FC 10 ;


END_ORGANIZATION_BLOCK 



